Tech Tip: 3M™ Crimplok™+ Fiber Connector Inspection Tips

November 2017

Light grey

Thin black line is visible

The 3M™  Crimplok™+ Connector and its installation tooling are designed to be easy to use for a wide range of skill levels in the field. However, visual confirmation of the end face of the connector is helpful in detecting potential problems that may have occurred in the termination process,

especially those mistakes caused by end user error. This Tech Tip document is intended to provide additional detail in identifying those issues and taking appropriate actions.

Caution reminder: Never look directly into the end of a fiber or connector when connected to a laser light source using the naked eye or a microscope.

Manual Scope Inspection

Inspect the fiber end face with 3M™ 200X View Scope 6365-VS (with additional angled adaptor 6565-VS/AS for APC connector) or similar optical view scope before the network has a laser light source feeding your connector. A digital scope with an indirect view screen may be used.

See below for examples of fiber defects and contamination.

Auto Scope Inspection

The 3M™ Crimplok+ connector requires different inspection zones when using an auto fiber inspection scope*.

- Please use the following zone parameters.

- Zone A: 0um – 25um

- Zone B: 25um – 85um

- Zone C: 85um – 135um

- Zone D: 135um – 250um

- Some auto inspection scopes will give a false passing result when the fiber is missing. Special attention should be used to determine if the fiber was short cleaved or not fully inserted into the ferrule.

- Visual appearance of the fiber is dark gray or black

- Difficulty focusing

- No surface details

- High insertion loss

(*3M™ has reviewed the use of Viavi P5000i with an OLP 82 power meter and found that they successfully detect most defects)

Ideal clean fiber end face

Examples of fiber defects

| Fiber End Face |  | Appropriate Action |
| --- | --- | --- |
| Missing fiber: - Appears as dark fiber - Difficult to focus - No surface details - High insertion loss | Dark grey No visible line. | Fiber may be short-cleaved or was not fully inserted into the connector. Take care to watch for bow in fiber to lift and movement of mark on fiber during tool actuation. → Cut off and re-terminate with new connector. |
| Missed polishing | Cleave nick mark Rough edge | ONT may detect signal, but insertion loss and reflection will both be high if fiber is not polished. → Complete the installation process using the 3M™ Crimplok™+ Nano-finisher. |
| Unacceptable chip ¼ diameter of fiber |  | Small chips and cracks on the end face are allowed. Maximum acceptable chip is ¼ of fiber diameter (toward center). Cut off and re-terminate with new connector. |
| Unacceptable crack |  | Acceptable chip example: |

Examples of contamination

| Fiber End Face Appropriate Action |  |
| --- | --- |
| Dirt or debris on fiber core | Cleanliness practice needs to be followed thoroughly when working with any fiber optic connectivity. → Full end face diameter needs to be cleaned using lint-free cloth and company approved cleaning fluid. |
| Water or cleaning fluid on ferrule | → Dry off using lint-free cloth and re-inspect the end face. |
| Cleaning fluid on fiber | → Dry off using lint-free cloth and re-inspect the end face. |
| Likely fiber thread caught | → Re-cleaning process is required. Apply pressure and twist connector tip on lint-free cloth. Re-inspect to get a visual confirmation. |

Other potential operator error examples

| Fiber End Face Appropriate Action |  |
| --- | --- |
| No water on polishing film during polishing step UPC shown in photo – contaminated surface with debris. APC not shown – Dry film will not polish correctly, but difficult to detect under visible inspection. | Both 3M™ Crimplok™+ Connectors APC and UPC require wet polish to achieve optimal results. → UPC and APC – Cut off and re-terminate with new connector. → Take care to spray lapping film with water before polishing step per instructions. |
| No lapping film in the 3M™ Crimplok™+ Nano-finisher during the polishing process. | → Re-polish per instructions. Take note to follow steps on placing the lapping film in the 3M™ Crimplok™+ Nano-finisher. |

Notes:

- The preferred cleaning method as listed in the instructions is to use isopropyl alcohol with lint free wipes.

- If necessary, after performing the preferred cleaning method, a “One-Click” style fiber optic connector cleaner, such as Fujikura One ClickTM can be used, by inserting the connector, performing a one-click cleaning, then rotating the connecter 90deg and perform a second one- click cleaning.

- Viavi modified inspection configuration can be downloaded at (3M?.com)

3M and Crimplok are trademarks of 3M Company.

### Important Notice

All statements, technical information, and recommendations related to 3M’s products are based on information believed to be reliable, but the accuracy or completeness is not guaranteed. Before using this product, you must evaluate it and determine if it is suitable for your intended application. You assume all risks and liability associated with such use. Any statements related to the product which are not contained in 3M’s current publications, or any contrary statements contained on your purchase order shall have no force or effect unless expressly agreed upon, in writing,

by an authorized officer of 3M.

### Warranty; Limited Remedy; Limited Liability.

This product will be free from defects in material and manufacture for a period of one (1) year from the time of purchase. 3M MAKES NO OTHER WARRANTIES INCLUDING, BUT NOT LIMITED TO, ANY IMPLIED WARRANTY OF MERCHANTABILITY OR FITNESS FOR A PARTICULAR

PURPOSE. If this product is defective within the warranty period stated above, your exclusive remedy shall be, at 3M’s option, to replace or repair the 3M product or refund the purchase price of the 3M product. Except where prohibited by law, 3M will not be liable for any direct, indirect, special, incidental or consequential loss or damage arising from this 3M product, regardless of the legal theory asserted.

### Communication Markets Division

6801 River Place Blvd. Austin, TX 78726-9000

Phone	1-800-426-8688

Web	www.3M.com/Telecom

Please recycle. Printed in USA © 3M 2017. All rights reserved. 78-0015-4252-7-A