Download instructions for 3M Crimplok + analysis profile with VIAVI OLP 82 power meter

- Download and install VIAVI FiberCheckPROTM onto your computer. This software can be found at viavisolutions.com

- Save the .pro file to your hard drive or other easily accessible place

- Go to Setup\FiberCheckPROTM Setup

- Click Analysis Profiles

- At the bottom left, click Show Advanced Options

- Click Import

- Navigate to the .pro file select it, and press OK

- Connect the OLP 82 to your PC or laptop using a USB converter. (USB-Micro to USB-A)

- In FiberCheckPROTM software go to the Setup menu and select OLP 82

- On the left- hand side, select Analysis Profiles

- All the available profiles will show on the left; anything already on the OLP 82 will show on the right.

- To add the profile to the OLP 82, select the profile on the left-hand side, then press the Copy->.

- You should now be able to select the profile on the OLP 82